SHED DETAILS

DIMENSIONS: 8’X10’

FLOOR

4”X4” PT RUNNERS

2”X6” PT FLOOR JOISTS

5/8 T&G OSB SCREWED INSTALLATION

WALLS

2”X4” STUDS 16” OC

DOUBLE TOP PLATE

½” OSB SHEATHING

ROOF

2”X4” RAFTERS

½” OSB SHEATHING

30LB ROOFING UNDERLAYMENT

25 YR OWENS CORNING ASPHALT SHINGLES

SIDING

DOUBLE 4” VINYL

WATER-RESISTANT HOUSE WRAP

TRIM

ALUMINUM & VINYL

PVC DOOR CASINGS

DOOR

60”X78”

BOARD and BATTEN

KNOTTY PINE T&G

3 COATS EXTERIOR POLYURETHANE

HEAVY DUTY 6” HINGES

WINDOW

24”X24” VINYL SLIDING w/SCREEN